STE6000

Key Features

Included Interface Options

None

Included Power

6 Outlet 120VAC Power Strip

Included Ventilation

120MM Honeycomb Vents

I/O Plates Included

3

Loading Style

TOP

Warranty

1 YEAR

Category

BENCH TOP

Absorber/foam

3/4" INSERTION LOSS

External Dimensions

37.5"(W) X 24.9"(D) X 17.4"(H)

Internal Dimensions

34.2"(W) X 22.2"(D) X 12.6"(H)

Shipping Requirements

Must Ship on A Pallet

Approx. Shipping Weight

100lbs

Shielding Effectiveness (dB)

Isolation At 2.4GHz

>85dB

Isolation At 5.8GHz

>80dB

INCLUDES THE FOLLOWING FEATURES:

•6 Outlet 120VAC Power Strip

•6 Pole Filtered Terminal Block feedthrough

•RF tight viewing window

•(2) ULTRA HIGH ISOLATION FLEXIBLE GLOVES GEN-3

•Designed for a wide variety of RF testing, function/s determined by user  customization.

•Perfect for (Wi-Fi 4, 5, 6, 6E (802.11a/b/g/n/ac/ax)

•Wireless device testing

•Bluetooth, RFID, 3G, 4G, 5G testing

•Totally customizable with a wide variety of I/O options available

The STE6000 is a blank enclosure ready to be customized to match your specific RF testing requirements. Our Engineering team is available to assist you to make sure your custom enclosure is designed to meet your unique RF testing requirements. This enclosure does incorporate a 120mm passive ventilation system with optional 12 volt DC, 115 VAC, or 230VAC fans available. As with all of our enclosures, your custom I/O configuration is brought out to a precision machined I/O panel, giving you off the shelf delivery of your custom order.

Features:

•Designed for WLAN testing

•Top loading swing away door

•Dual ventilation standard or fan forced cooling optional.

•Exclusive double lip RF tight gasket

•Universal I/O connector interface plate

•Insertion Loss RF absorbent foam liner Standard

•Reflection Loss solutions available

•Optional security locking latch available

Isolation (Base Enclosure, may vary depending on I/O or options selected in final configuration):

•>85dB to 3GHz

•>80dB to 6GHz

I/O Connectors:

•See options for off-the-shelf connectors/interfaces available.

•Coaxial connectors: SMA, BNC, TNC, N, N to SMA, etc..

•Digital interfaces: USB, Ethernet, Video, Serial, Audio, Fiber, etc..

•Power filter interface: AC and DC

•Power Strip Options: US, UK, European, France, Swiss, Italy, Israel, India, China, Australia, Multi Country Config, etc…

•Custom connection options can be sourced through our Engineering